PCD 1 (3)

GROUND MOVEMENTS (3)

# Taxiing instructions

Note: In ICAO and UK phraseology, taxi is used for aircraft and proceed for vehicles.

(to a/c) . 	 Taxi straight ahead to Delta 4.

(to vehicle) . 	 Proceed straight ahead to Bravo 5.

 Continue straight ahead to Papa 20.

 Pass the 757 at the holding point at own discretion / you are n°1 to depart.

.		 Depart the apron and taxi to the hangar / watch out for the STOP signs at the intersection

 Follow the A300 that is taxiing out to runway 25.

 Give way to the DC10 that is taxiing in to the south terminal.

 You can taxi back to the apron via taxiway n°5.

 You'll have to turn round at the north end of the runway.

 Taxiway n°6 is closed / you'll have to backtrack the runway and turn on to taxiway n°1.

 You go beyond the DC9 that's on your left and then you turn right.

 Please expedite taxi / taxi to the hangar direct.

 Taxi slower / caution for the 767 to your left.

 Do you want to be towed to Delta 6 ?

 Follow the yellow line to your left along the grass strip.

 Left turn from your present position / taxi north for departure runway 13.

 Please pull in to the left / a 146 is going to pass you.

 Don't worry about the 737 catching you up / he is on his way to the maintenance area.

 l don't think you've got enough space to overtake the Cessna.

 Taxi via taxiway number seven to holding point runway 26/report approaching the stop-bar.

 Caution the Cessna taxiing towards you from your left.

 Taxi with caution / taxiways are slippery / taxi via north taxiway.

 Taxi to Charlie 14 via the loop taxiway.

 Taxi eastbound from your present position and I'll keep you advised.

 You can release tail parachute after next turn.

(to vehicle) .       Proceed on mid-taxiway to executive terminal.

------------

PIL 	 I'm not familiar with your airport / could you please guide me to the runway-in-use ?

------------

PIL 	British Caledonian 887 / ready to taxi.

CTL 	B-Cal 887/ taxiways 4, 5 and 1 to holding point Echo / keep well to the left passing the North Park, due work in progress.