- PCD 11-2

- METEOROLOGY (2)

- En route met information to VFR aircraft

- CTL :	The met office (US: weather bureau) reports that radar doesn't show anything up to the Crowfield area at present / you will encounter ceiling 4 octas, 3 to 4000 / there are a few thunderstorms, light rain showers in the Harold area and I’ve just got the Crowfield present weather : cloud (or ceiling) 7 octas, 5500/ visibility 4, with light rain / they're reporting a strato-cumulus bank northwest / temperature 21°: dew-point 9° / wind south south-east 19 with gusts to 25 Kt.

------------------------------------------------------

- PIL :	Request weather information.

CTL:	Roger / there is a squall line expected to develop in the vicinity of Newton moving eastward about 30 Kt / moderate to locally severe turbulence spreading along the squall line with strong and gusty surface winds / they expect a tornado in the Warton area around 3 miles diameter / surface winds with gusts of 65 / extreme turbulence / scattered cumulo-nimbus with maximum tops to 50 000 / this extends to 50 miles north-west of Mansfield.

PIL :	Roger / do you think we can make it to Linton by heading due east for a while in order to avoid the activity and then head north?

CTL :	Affirm / stay as close to the Ashwell area as possible without deviating from your course / you might run into turbulence depending how close you get to the thunderstorm activity.

------------------------------------------------------

- PIL :	95 Tango over the Marston VOR at 10 / 9500 ft VFR to Weston / estimating Baker at 51 / Underwood next.

CTL :	95 Tango / roger / it looks like flight conditions are getting pretty rough in the Weston area at the present time / over Marston now we' re getting ceilings 8 octas 3300, visibility 12 / I’ve got some reports coming on the teletype / stand by one.

PIL :	95 Tango.

CTL :	There's a chance of hailstorms beyond Baker north of your course / Baker is 4 octas, 2300/7 octas. 5 000.

PIL :	Roger / we're on top of a solid overcast / the tops are about 7500  increasing towards the north and the west.

CTL :	How do you find the flight conditions up there? do the tops look smooth?

PIL :	Air is very smooth and the tops are extremely stratified.

CTL :	Thank you.

PIL :	When do you think we'll be able to get below the overcast?

CTL :	This layer of clouds over Marston at the present time shouldn’t extend too far towards the north / you'd better proceed via Columbia rather than Baker .

PIL :	Roger.

------------------------------------------------------

- PIL : 	What's the weather like beyond Foxley?

CTL :	The met office reports: moderate to severe turbulence and moderate to severe icing in the clouds, effective from 2000 to 3000 ft / solid cloud from 5000 to 20000 ft / we have a pilot report from Foxley : he reported moderate to heavy icing in the clouds and moderate turbulence / the aircraft was a Twin Beech.

PIL :.	Roger / what kind of freezing level have I got on the Charlton-Oakley route ?

CTL :	The freezing level in the Foreman area runs about 6000 to 6500 lowering ( or : sloping downward or slanting) slightly to about to 3500  to 4500 in the Kent area / icing's very light.

PIL : 	Roger / thank you.

------------------------------------------------------

- CTL :	Say flight conditions.

PIL : 	VFR on top.

------------------------------------------------------

- CTL :	Say flight conditions

PIL :	IFR in and out of cloud.

------------------------------------------------------

- PIL :	Conditions are very bumpy at this level / can we have FL120 ?

CTL :	Negative. there's traffic right with you at 120/ I can give you lower, say 100 / would you like to make a CAT* report ?

------------------------------------------------------

- PIL :	We were going to Beagle but we've got above a sheet of stratus, and l'm not IMC rated.

CTL :	Roger / what's your fuel state?

PIL :	About 40 minutes left.

CTL :	Remain this frequency / I check with Met for a break in the cloud.

*CAT: clear air turbulence.