PCD 2 (1)

GROUND MOVEMENTS ( 6 )

Parking and ground-handling problems ( a )

1	PIL 	Could you confirm whether our stand is Papa 20 or 21, because l think they expect us on 21.

CTL 	No, it's the Airbus behind you that's expected on 21.

------------

2	CTL 	Taxi slowly / I’ll call you back with a stand / the apron is a bit crowded at the moment.

PIL 	Roger.

(Time interval)

CTL 	Stand Charlie 15 / taxi with caution after passing the intersection due work in progress.

------------

3	CTL 	Taxi to Alpha 11 / caution on approaching your stand / there's no marshaller available due to

ground staff strike.

------------

4	CTL 	Please disregard the marshaller's instructions and pull in a bit closer to the DC 9 / caution the blast fence / sorry, but it's a bit of a squeeze on the apron this morning.

------------

5	(Aircraft has just come on to the frequency after landing).

PIL 	Ground / GAPSL / runway vacated.

CTL 	GSL / taxiways 1, 7, 6 and 4, to the General Aviation Terminal / they’ve put you down for stand 6 but l think that's wrong.

PIL 	We ' d like to use stand 6 / could you ask your management to confirm, the stand number ?

CTL 	They've confirmed it by phone, taxi to six.

------------

6	PIL 	Ground / GSL / stand six seems to be blocked by vehicles / all three of them are parked on the stand, in my way.

CTL 	OK, GSL / I’ll get a van out for you / if they don't move, keep me advised.

------------

7	PIL 	Ground / DANAIR 521 is taxiing in.

CTL 	Roger, DANAIR 521 I taxi to the junction of one and six, to hold there, due to apron congestion.

------------

8	( The aircraft has just landed and vacated the runway) .

PIL 	Ground / KLM 402 is on the main taxiway.

CTL 	Roger, KLM 402 I taxiways 1, 5 and 4 to stand 23.

PIL 	Confirm the stand number.

CTL 	Er, stand 23 I KLM 402.

PIL 	We .would prefer a stand on the south pier / could you change that ?

CTL 	Stand-by / I’ll contact the apron controller *.

(Time interval)

CTL 	KLM 402 I Ground / what was the reason for your request ?

PIL 	We' ve got a problem with the aircraft / we’d like to be near to our maintenance area.

CTL 	Roger / I’ll call you back.

(Time interval)

CTL 	KLM 402 / taxiway 1 all the way to stand 8.

PIL 	Thank you very much.

_________________________________________________________________________________

( * )An airport employee (not an ATCO) who allocates stands according to ACFT SIZE engineering and customs requirements.