PCD 4 ( 1 )

APPROACH AND LANDING ( 5 )

A ) Approach instructions

1 	CTL 	Swissair 807 / l say again / Swissair 807 / do you read ?

PIL 	Approach / Swissair 807/ read you five.

CTL 	807 / did you read my previous transmissions ?

PIL 	Negative / London has only just put us over to you.

------------

2 	CTL 	PA / make a right turn / the long way round to roll out heading 320°, delaying action.

PIL 	Roger / PA is going RIGHT on to 320°.

| “Turn left heading 320°” | “ Turn right, I say again right, heading 320°. OR “Turn right the long way round heading 320°” | “Orbit right” “Make a 360° turn in your present position” |
| --- | --- | --- |

3	CTL 	Beatours 461 / turn right heading 020° for spacing / unidentified traffic / 11 o'clock 7 miles / height unknown / will pass behind you on that heading.
( When 461 has turned on to 020°, the unidentified traffic wi11 pass behind him )

PIL	Roger / Approach / we've got him in sight / looks like a big jet, well above us.

CTL	461 / if you're happy to avoid him visually, you may stay on your present heading / you're n°1 to land.

-------------

4	PIL 	KLM 507 has the field in sight.

CTL 	Roger 507 / continue visually keeping clear of built-up areas.

PIL 	What number am l in traffic ?

CTL 	507 / you've just become n°1 / continue approach / contact Tower on 119.45.

-------------

B) Speed control

5 	CTL 	SAS 885 / you are 12 miles from touchdown / n°2 to land / ten miles behind n°1 / can

you increase speed for a short while ?

PIL 	885 / what speed would you like ?

CTL 	Can you give me 200 kt to the outer marker ?

PIL	 885 / we'll do our best.

-------------

6 	CTL 	DANAIR 1001 (one thousand and one) / you're a bit close to the aircraft ahead /

continue heading 350° / I'll take you through the localizer course and after this slight delaying action, you'll be closing the localizer from the right.

PIL 	one zero zero one is doing 180 kt / would it help if we reduced to 150 ?

CTL 	Negative one thousand and one / there is further traffic behind you.

-------------

7	CTL 	GPL / you're nine miles from touchdown / say speed (*).

PIL 	PL / 120 Kt.

CTL 	PL / can you maintain (keep up) that speed to the outer marker ?

PIL 	PL will fly at 120 Kt to the outer marker.

CTL 	Thank you PL / break break / DANAIR 475 / you are 12 miles behind a slow aircraft / if you could start reducing speed now to 170 Kt, it will save me asking you for a bigger speed reduction later on.

PIL 		DANAIR 475 is coming back to 170 Kt / we can make it slower if you wish.

CTL 	Negative / (make your) speed 170 Kt to the outer marker.

PIL	 Wilco / DANAIR 475.

-------------

8 	CTL 	DANAIR 4201 / turn left heading 310° / you are closing the localizer from the left /

report established.

PIL 	4201/ left 310 / what's my distance from touchdown ?

CTL 	4201 / 12 miles from touchdown / speed at your discretion (or to suit yourself) / does that check ?

PIL 	Affirm / my DME indicates just under twelve miles.

**French phraseology recommends the use of say speed instead (If report speed which is the standard UK/ICAO phrase.