PCD 4 (2)

APPROACH AND LANDING (6)

# Zone traffic

1          PIL 	Headland / this is GBEEN / a Cherokee on a cross-country : Tinningslea, Kellyfield, Cambourne, Tinningslea / VFR at 1.500 feet / estimate Kellyfield at 1521 / can l remain on this frequency for traffic information ?

CTL 	GEN / affirm / advise me of any changes of level or flight conditions. (= VMC / IMC).

(Time interval)

PIL 	Headland / GBEEN is approaching Cambourne.

CTL 	GEN / I've got nothing further for you / contact Cambourne on 119.8 / revert to this frequency when you've finished with them.

(Time interval)

CTL 	GEN / could you turn on to a south-westerly heading and maintain VMC ? / this is to keep you clear of scheduled jet traffic .

PIL 	Affirm / can you give us an idea of where he is ?

CTL 	Traffic is a 146 / 1 o'clock / 5 miles / same level / crossing you right to left.

------------

2 	PIL 	Headland / GATPN / overhead Kellyfield.

CTL 	Roger, GPN / identified on handover from Aldon Radar / say level (*)

PIL 	PN is at 2.000 feet.

CTL 	Roger, PN / there is considerable traffic in the circuit at Kellyfield / l wanted to be sure you were above it I you have 15 miles to run to Headland I continue on your present heading and report the field, in sight.

PIL 	We've got information Delta / could you confirm the QFE ?

CTL 	Information Delta is current I the QFE 1010.

-----------

3 	PIL 	Headland / GPA / a Cessna 150 / from Kellyfield to Cambourne / request a VFR

crossing of your zone.

CTL 	Roger, GPA / continue towards the Headland zone from Kellyfield / I'll call you back with a clearance / QNH is 1022.

-----------

*French phraseology recommends the use of say level instead of report your level which is the standard UK/ICAO phrase.

4 	PIL 	Headland Approach / GBELS / a student pilot on a cross-country from Tinningslea to

Cambourne / request permission to cross your zone VFR.

CTL 	GBELS / what is your present position ?

PIL 	I'm just on the northern boundary of your zone / heading 170° at fifteen hundred feet VFR.

CTL 	Roger, GLS / you're cleared to cross the zone V FR / report the field in sight.

(Time interval)

PIL 	GLS has the field in sight.

CTL 	Roger / would you cross the field over the threshold of runway 24, to keep you clear of departing jet traffic on runway 24/ report over the field.

PIL 	Wilco / Headland / I’ll give you a call over the 24 threshold.

-----------

5 	PIL 	Headland APP / GAXUS / V FR flight from Tinningslea to you / do you have my

details?

CTL 	GUS / Headland / yes, I've got your plan / it will have to be special VFR due to the weather / you are cleared from the zone boundary to the field, special VFR not above 1.500 feet / report the field in sight.

PIL 	Roger / GUS is cleared special VFR not above 1.500 ft / call you visual (= field in sight ).

(Time interval)

CTL 	GUS / there is traffic to you, not on frequency, but believed to be a Cessna 172 at 1.500 ft / 10 o'clock / 2 miles / crossing you left to right.

PIL 	Roger / Headland / l have him.

CTL 	GUS / call Tower on 118.1.

------------

6	PIL 	Headland / GBEKS is from Cambourne for Kellyfield / have you any traffic ?

CTL 	KS / nothing for you / suggest you proceed direct to Kellyfield.