PCD 9 (1)

# RADIO AND NAVAID FAILURES

A) POOR TRANSMISSION :

PIL 	London Control / Whiskey Bravo is leaving FL 250 for 200.

CTL 		Whiskey Bravo / London / your transmission is partly broken up / understand leaving FL240 / is that correct?

PIL 	Negative / we're leaving 250, I say again 250.

CTL 	Whiskey Bravo / roger.

B) RADIO FAILURE :

1) 	CTL 	KLM 318 / London I climb to FL 260 / report reaching.

..... (no answer) .....

CTL 	KLM 318 / London / do you read?

.....(no answer) .....

CTL 	KLM 318 / London / reply not received / if you read : squawk 6707, I say again squawk 6707.

(Time interval)

CTL 	KLM 318 / London / you are identified .



2) 	PIL 	(unreadable)

CTL 	Sabena 621/ London / I don't read you / if you read, turn left heading 270°, I say again, heading 270°.

(Time interval)

CTL 	Sabena 621 / London / turn observed / position: 15 miles west of Woburn / will continue to pass instructions.

C) RADAR TROUBLE :

1 ) 	CTL 	Lufthansa 862 / Lackhead Centre / radar contact / cleared to the Renton intersection

heading 240.

PIL 	Lackhead Centre / Lufthansa 862 / cleared to Renton / heading 240.

CTL 	Lufthansa 862 / Lackhead / radar contact lost due to ground clutter / will advise you when contact regained / resume own navigation to the Renton intersection.

PIL 	Lackhead / Lufthansa 862 / resuming own navigation to Renton.



2) 	CTL 	Radar unserviceable / I can no longer assist you / reduce speed by 30 kt to increase separation.

PIL 	Roger.



3) 	PIL 	Request. vectors until past Mansfield.

CTL 	I won't be able to show you that far, 'cause my scope only goes out to 70 miles / I’ll offcenter the display (US : I’ll decenter).to see if I can keep you a bit further .



4) 	CTL 	I've got a blind spot on my radar / I won't show you again until 5 to 10 miles past Bryce / continue present heading / report Bryce.

PIL 	Roger.



5) 	CTL 	Our radar is operating on low power at the moment / we're unable to spot primary targets / adjust speed so as to cross Gulfport at 25.

PIL 	Roger.



6) 	CTL 	Cleared to Heston direct / FL 210 / squawk 6736.

PIL 	Roger / squawking 6736.

CTL 	Check transponder / squawk reply not received.

PIL 	Transponder's inoperative.



7) 	CTL 	I’ve lost your squawk once again / would you recycle 6532.

PIL 	Squawking 6532.

D) VOR UNRELIABLE:

1) 	PIL 	Passing the Bradford VOR I FL 160.

CTL 	I show you 10 miles north of Bradford I can you check your receiver?

PIL 	Our receiver has switched to "from" I it seems to be working normally.

CTL 	The Bradford VOR must have gone wrong again / I’m going to alert maintenance.



2) 	PIL 	We're not getting any reception from the Charleston VOR I can you check whether it’s on the air?

CTL 	Stand by one.

(Time interval)

CTL 	Maintenance confirm that the Charleston VOR is out of service I we can give you radar assistance until 50 miles north of Charleston.



3) 	PIL 	Request FL 80 over the Maple VOR.

CTL	FL 80 not available due to VOR calibration check in the Maple area I climb to FL 100.

PIL 	Roger.



4) 	PIL 	We'll have to go around.

CTL 	Roger I climb straight ahead to 3 000 feet and contact approach on 126.7.

(Time interval)

CTL 	What was the reason for the missed approach?

PIL 	Well, firstly the glide path was fluctuating wildly, then the DME went «U/S»* /we didn't receive the outer marker and the cloud is much lower than reported / can you check the DME, glide path and outer marker at your end before we make another attempt to land?



*"U/S" : spoken "YOD ESS" in the UK for unserviceable.